FastCAD(R) Vector Font Files				8/26/88

-----------------------------------------------------------------------------

The font file format has been changed.  The new one is called "Format 2".

Old .FDF files can be made compatible by adding the header line:

#48,63,15,0,0,16,0

as the first line of the .FDF file, and by removing any trailing

* lines from the end of the file. (The last line of text should

be the last vector defined for the last character). Recompile

the font and the result will be compatible with FastCAD &

EasyCAD versions 1.30 and above.

-----------------------------------------------------------------------------

To compile a font definition source file (.FDF) into the useable

binary font file (.FNT) you may use FONTCOMP:

FONTCOMP fontname

This will compile FONTNAME.FDF into FONTNAME.FNT.  If no name is

specified, STDTEXT is assumed.

-----------------------------------------------------------------------------

The .FNT binary font file format is:

1st: Byte MAXVX(0:255) table of maximum horiz vector (character width)

The first byte is the font file format id (=FCFmt)

2nd: Integer OFSET(0:255) offset from start of file of vector list

for character having the index value in ASCII 1..255.

The offset for character 0 points to the font specs, a table

of 7 integers in the following order:

1: # vectors in txt body height.  This defines the text height.

2: Maximum vector value (normally, top of text body).

3: Text Baseline Vector. Text is positioned along this vector.

4: Minimum vector value (normally, lowest descender).

5: Recommended # vectors intercharacter spacing (horizontal).

6: Recommended # vectors interline spacing (vertical).

7: Special handling flags (normally 0).

bit 15=1 if font uses arcs. (set by program, not #specs).

bit 14=1 if plotter font name info follows

8: Word = plotter font height % of spec height n/1000.

(needed because fonts in point sizes are < point equiv high)

9: 40-byte max length (not incl 0) ASCIIZ string plotter font name

3rd: Byte VECTABLE(0:16383) the table of byte vectors:

Line move & draw vectors are:

| 7 |  6  5  4  3  2  1  0 |     x vector first

| d | x6 x5 x4 x3 x2 x1 x0 |

| 7 |  6  5  4  3  2  1  0 |     y vector second

| 0 | y6 y5 y4 y3 y2 y1 y0 |

and   d=1 for draw, 0 for move.

Arc draw vectors are:

| 7 |  6  5  4  3  2  1  0 |     x center first

| 1 | x6 x5 x4 x3 x2 x1 x0 |

| 7 |  6  5  4  3  2  1  0 |     y center second

| 1 | y6 y5 y4 y3 y2 y1 y0 |

| 7 |  6  5  4  3  2  1  0 |     radius third

| 0 | r6 r5 r4 r3 r2 r1 r0 |

|  7  6  5  4  3  2  1  0 |      start angle forth

| s7 s6 s5 s4 s3 s2 s1 s0 |      0..255 are 1/256 of circle, draw ccw

|  7  6  5  4  3  2  1  0 |      end angle fifth

| s7 s6 s5 s4 s3 s2 s1 s0 |      0..255 are 1/256 of circle, draw ccw

The end of a character is marked by:

| 7  6  5  4  3  2  1  0 |       A byte = 0FFh

| 1  1  1  1  1  1  1  1 |

-----------------------------------------------------------------------------

The .FDF Font Definition Source File Format is:

1 - The first line start with the # character and lists the values

of the seven font specifications:

#1,2,3,4,5,6,7

1: # vectors in txt body height.  This defines the text height.

2: Maximum vector value (normally, top of text body).

3: Text Baseline Vector. Text is positioned along this vector.

4: Minimum vector value (normally, lowest descender).

5: Recommended # vectors intercharacter spacing (horizontal).

6: Recommended # vectors interline spacing (vertical).

7: Special handling flags (normally 0).

bit 15=1 if font uses arcs. (set by program, not #specs).

2 - Character specifications follow. Each starts with a header line:

*value  comments

where the first character on the line is * and value is the

numeric value of the character.  A leading zero denotes a

hexidecimal value.  If comments follow, they must be separated

by one or more spaces.

One or more vector definition lines follow this to define

the character.  Valid vectors are MOVE, DRAW, and ARC.

The order of the vector definitions determine the order

of drawing - pay attention to routing efficiency.  Multiple

moves will be removed, and only the last move in any

multiple-move sequence will be compiled. The character

definitions do not have to be in any order.

3 - The move vector is:

Mx,y

The first character must be an upper-case M. The "pen" will

move without drawing to the specified cell position.

4 - The draw vector is:

Dx,y

The first character must be an upper-case D. The "pen" will

draw from the current position to the specified cell position.

5 - The arc vector is:

Acenx,ceny,radius,startangle,endangle

The first character must be an upper-case A. The start and end

angle are specified in whole degrees and will round to the

nearest 22.5 degree multiple (16 possible angles).  The current

"pen" position is unchanged.

-----------------------------------------------------------------------------

A word about intercharacter spacing:

A font may be designed to either specify the intercharacter

spacing globally for the font, or by setting intercharacter

spacing to zero, use initial and final "move to" vectors

to indicate the character spacing. STDTEXT uses the first

technique, while FANCY and SMOOTH specify the spacing

differently for each character.

-----------------------------------------------------------------------------

The Beta-Test FMAKER 0.04 program:

The XP-module FMAKER is an interactive font editor.  You copy

both FMAKER.COM and FMAKER.MNU into your main FastCAD directory

and use the command "CONFIG E" to install it.  You should not

leave it installed when not doing font work, as it is large (20K)

and performs automatic load scans and initializations that will

slow down other work.  Leaving it installed will do no harm to

non-font drawing files.

FMAKER will only create and work with "Format 2" .FNT files.

To use FMAKER, start FastCAD without loading a drawing by

entering the DOS command "FCAD".

Once FastCAD is ready, type the command "FONT" to select a

font drawing, or to make a new one.  You will see the standard

drawing selection screen - select an existing font drawing or

enter the name of a new one to be created.  The XP module will

then automatically set up the conditions for defining fonts and

display a working character cell grid.

Next, a font specifications dialog box is displayed, showing

the current font header record parameters.  You can click

on any item and then type in a new value.  Click the OK item

to continue. (The SPECS command described below lets you

change these values later).

FMAKER places the working grid on layer 0 and freezes it so that

it will not be selected when editing.  Each character is defined

by placing lines, arcs ,and points on the layer that has the

number equal to the characters' 8-bit extended ASCII value.

The DRAW menu contains the following items:

Change Key

Line

Arc

End Point

Select change key and press the key or ALT-numKeypad combination

for the character you wish to define or edit. FMAKER will then

make the selected characters' layer visible and current, and all

other layers (except 0) invisible.

The grid has tick marks at the highest, lowest,baseline, and

topline Y values for assistance. (The highest and topline values

are usually the same.)  All characters start out at the baseline

and leftmost position.  Draw the character using lines and arcs.

The order in which they are placed determines the order in which

they will be drawn. Since this can make a difference in the

text drawing speed, you may wish to use the FRONT command to

reorder things when you have finished designing your character.

Only entities drawn in color #1 (green) will be used by FMAKER.

Other colors may be used for planning, construction lines, etc.

You can use a point to mark the rightmost position for the

character, or the start of the next character if the headers'

character spacing is set to zero.

When you have finished designing or editing your font, SAVE

the drawing and use "COMPILE" from the FONT menu.  This will

compile and write the binary .FNT file with the same name

as your font drawing.

To use the new font file, you must quit FastCAD and restart

it. If necessary, use "CONFIG F" to install the font and

"TSPECF" to select it, then draw some text with the "TEXT"

command.  Do not attempt to load and use the font without

first quitting FastCAD.  If you had laready configured that

font, FastCAD would still be using the old version that

it initially loaded, and not the newly-compiled version.

The FONTS menu also has an IMPORT command.  You may use this

to automatically make a FastCAD .FCD drawing file from an

existing .FNT binary font file.  Select the IMPORT command

from the FONTS menu.  You will see the standard file selection

box listing any .FNT files available.  Select one.  Next you

will be asked to confirm that it is ok to discard the present

drawing. Press "Y" to do this.  The font will be imported

into drawing format, and you will then be asked for the name

of the new drawing file.  If you do not pick one, WORK.FCD

will be used.  You can now edit and save the new drawing file

and compile it for revised .FNT output.  You should not

use the same name for the changed font as the one that was

imported.

The FONTS menu also has a SPECS command.  This brings up the

font specifications dialog that is used when a new font is

initialized (as with the FONT command).  Clicking on any

value box will allow you to type in a new value.  When you

have made any changes, click the OK box.

Set character spacing to 0 if your font defines the spacing

between characters as part of each character definition, or

the desired standard spacing if not.

The maximum value of 127 and minimum value of 0 applies to

the maximum, minimum, baseline and top positions.  The topline

value is the baseline+character height.  The number of

vectors specified for height are scaled to fit the real

text height when drawn.  Text is placed by indicating a left

center or right position on the baseline.  maximum and

minimum values are specified to give you a grid allowing

ascenders and descenders.

The compiler makes any changes needed to the flags value.

-----------------------------------------------------------------------------